GUJARAT POLICE INNOVATION HACKATHON 2026

Technical Proposal — High-Level Design (HLD)

Unified CCTV Integration, AI Video Analytics, Watchlist Correlation and Cross-Camera Intelligence

Proposed Architecture: Hybrid Model 5
(Model 1 + Model 3 + selective Model 2)

# Document Control

| Item | Value |
| --- | --- |
| Problem Area | Statewide CCTV integration, AI analytics, watchlist correlation and intelligent alerts |
| Proposed Model | Hybrid Model 5: Model 1 mandatory + Model 3 federation + selective Model 2 direct stream integration |
| Core POC | Authorized Sentinel/government CCTV feeds plus participant-owned demonstration environment |
| Future Scale | Approximately 80,000 registered cameras with regionalized processing and horizontal scaling |
| Backend | Python + FastAPI |
| Frontend | React |
| Streaming | RTSP over TCP for AI; HLS/WebRTC for visualization where authorized |
| AI | PyTorch + OpenCV + FFmpeg/GStreamer |
| Messaging | Apache Kafka |
| Database | PostgreSQL + PostGIS |

# Executive Summary

The proposed platform creates a unified, secure and interoperable intelligence layer above existing departmental CCTV ecosystems. It does not require immediate replacement of departmental cameras or VMS platforms. Model 1 is the mandatory foundation for statewide camera registry and GIS metadata. Model 3 provides a vendor-neutral federation layer using adapters, APIs, SDKs and standard protocols. Model 2 is applied selectively where direct RTSP/ONVIF/API access is available and operationally appropriate.

The platform ingests camera metadata, establishes authorized stream connectivity, performs vehicle detection and tracking, selects high-quality frames, detects number plates, executes ANPR/OCR with multi-frame consensus, records evidence-backed observations, compares eligible recognitions against active authorized watchlists, distributes events through Kafka, and exposes investigation workflows through FastAPI and a React command centre.

The design deliberately separates media PTS, source/calendar time and application ingestion time. Camera-local PTS values are not treated as a global clock. Cross-camera chronology is established with a validated common source-time mapping exists. Verified camera coordinates are used for GIS; coordinates are never fabricated.

# Contents

1. Problem Statement and Understanding

2. Objectives and Requirements

2.1 Submission Requirement Coverage Matrix

3. Integration Model Selection and Justification

3.1 Model 1 - Mandatory Camera Registry and GIS

3.2 Model 2 - Selective Direct Stream Integration

3.3 Model 3 - Federation Middleware

3.4 Final Hybrid Selection

3.5 PoC Scope vs Statewide Production Scope

4. High-Level Architecture

4.1 Workflow-to-Architecture Model Mapping

4.2 End-to-End Workflow

4.3 Component Interaction and Data Flow

4.4 Departmental Infrastructure Retention and Central Intelligence Layer

5. Camera Registry and GIS Foundation

6. CCTV Camera, NVR and VMS Interoperability

6.1 Integration Contract

6.2 Rules

7. Stream Ingestion and Processing

7.1 Continuous Coverage vs AI Sampling

7.2 Timing

8. AI Analytics Pipeline

8.1 AI Analytics Capability Matrix

9. ANPR/OCR and Multi-Frame Consensus

9.1 AI Evidence Chain

10. Observed Vehicle Intelligence Layer

11. Watchlist Architecture and Matching

12. Kafka Event Pipeline

13. Alert Generation and Notification Workflow

13.1 Alert Rules and Eligibility

13.2 Alert Prioritization

13.3 Alert Lifecycle and Operator Interaction

13.4 Notification and Evidence Presentation

14. Cross-Camera Correlation and Journey Reconstruction

14.1 Time Model

14.2 Journey States

14.3 Spatial Correlation

14.4 Observation Graph

15. PostgreSQL/PostGIS Data Architecture

16. React Command Centre

17. APIs and Integration Contracts

18. Security, Privacy, RBAC and Auditability

19. Network and Bandwidth Planning

20. Storage and Retention Strategy

21. Scalability to Approximately 80,000 Cameras

21.1 Capacity and Scaling Model

21.2 Geo-Distributed Regional Processing

22. Reliability, Disaster Recovery and Business Continuity

23. Deployment Architecture

24. Performance and Capacity Planning

25. Testing and Evaluation Strategy

26. PoC and Government Feed Demonstration

26.1 Participant-Owned Demonstration

26.2 Government-Provided Feed Demonstration

26.3 Supporting Technical Evidence

27. Technical Prerequisites and Department Inputs

27.1 Department Integration Readiness Checklist

28. Risks and Mitigations

29. Expected Operational Benefits

30. Conclusion

# 1. Problem Statement and Understanding

Multiple Government departments operate independent CCTV ecosystems with different camera technologies, VMS platforms, storage arrangements, retention policies and network environments. The challenge is to integrate public-domain government feeds into a common operational ecosystem while retaining departmental systems and supporting future integration of eligible private/public-facing cameras where permitted.

Diversified sources: IP cameras, analog systems through gateways, departmental VMS/NVR systems and other vendor-managed environments.

Non-uniform media: different codecs, resolutions, bitrates, frame rates and network behavior.

Operational fragmentation: separate viewing interfaces and departmental workflows.

Intelligence fragmentation: critical databases such as VAHAN, SARTHI, eGujCop, AFIS and NAFIS are not automatically correlated with CCTV events.

Need for real-time intelligence: identify vehicles/persons/objects, correlate with authorized watchlists and surface actionable alerts.

Need for statewide scale: architecture must support incremental onboarding and future expansion toward approximately 80,000 cameras.

# 2. Objectives and Requirements

| Requirement | Target Design |
| --- | --- |
| Unified inventory | Model 1 camera registry with PostgreSQL/PostGIS and GIS APIs |
| Heterogeneous integration | Model 3 adapter/federation layer plus selective Model 2 direct access |
| Live analytics | Distributed AI workers consuming authorized RTSP/HLS streams |
| ANPR | Vehicle/plate detection + OCR + multi-frame consensus |
| Watchlists | Authorized active lists; synthetic demo data for PoC |
| Real-time events | Kafka topics with schema, retries, DLQ and idempotency |
| Vehicle tracing | Observation-based cross-camera correlation; journey only when time/spatial evidence permits |
| Security | RBAC, least privilege, encryption, auditability and secret hygiene |
| Scalability | Horizontal stream/AI workers, partitioned events, scalable API/database tiers |
| Operator interface | React command centre with map, video wall, vehicle search, journey, watchlist and alerts |

# 3. Integration Model Selection and Justification

## 3.1 Model 1 — Mandatory Camera Registry and GIS

Model 1 is the mandatory baseline. It establishes the authoritative inventory of cameras and metadata and provides the common identity used by stream connectors, analytics, monitoring, APIs and the command centre. It supports bulk/manual/API onboarding, search, health state, GIS mapping, coverage analysis and audit trails.

## 3.2 Model 2 — Selective Direct Stream Integration

Model 2 is used where a department or sandbox exposes direct RTSP, ONVIF, vendor APIs or other authorized access. It is especially appropriate for AI inference because the analytics service can consume an accessible stream without changing the source VMS.

## 3.3 Model 3 — Federation Middleware

Model 3 is the primary interoperability layer for heterogeneous VMS environments. Connector/adaptor modules isolate vendor specifics from central services and provide a normalized stream/event/metadata interface. Departments retain their existing infrastructure and operational control.

## 3.4 Final Hybrid Selection

Hybrid Model 5 = Model 1 (Registry/GIS) + Model 3 (Federation) + selective Model 2 (Direct Stream Access)

Model 1 provides the mandatory statewide source of truth for camera assets.

Model 3 handles departmental/VMS heterogeneity and future vendor onboarding.

Model 2 provides an efficient direct path for cameras with standard/available stream access.

The hybrid model architecture avoids an unnecessary rip-and-replace strategy while still enabling centralized analytics and operations.

The design can later incorporate scalable infrastructure without changing the integration model.

3.5 PoC Scope vs Statewide Production Scope

The PoC demonstrates the core end-to-end vehicle intelligence path while the HLD defines production-ready extension points required for statewide deployment. Capabilities are therefore categorized explicitly as implemented, architecture-ready or future extension.

| Capability | Current PoC | Statewide production direction |
| --- | --- | --- |
| Camera registry / metadata | Implemented / integration-ready | Central authoritative registry with departmental onboarding |
| Sentinel RTSP integration | Implemented | Regional connector pools for heterogeneous sources |
| Vehicle detection and tracking | Implemented | Distributed CPU/GPU AI workers |
| Plate detection + ANPR/OCR | Implemented | Distributed analytics with model lifecycle and benchmarking |
| Watchlist matching | Implemented with synthetic representative data | Authorized departmental watchlists and policy-controlled matching |
| Kafka event pipeline | Implemented / integration-ready | Clustered, partitioned event backbone |
| Cross-camera vehicle correlation | PoC | Statewide correlation with validated source-time and GIS metadata |
| Person / FRS analytics | Architecture-ready | Dedicated person/face analytics subject to policy and approvals |
| High-availability database | Production direction | HA PostgreSQL/PostGIS with backup and recovery controls |
| Command centre | Implemented / in development | HA web tier with role- and department-scoped access |

# 4. High-Level Architecture

GOVERNMENT CCTV SOURCES
                                │
              ┌─────────────────┼─────────────────┐
              ▼                 ▼                 ▼
          Department VMS     RTSP / ONVIF      HLS / WebRTC
              │                 │                 │
              └─────────────────┼─────────────────┘
                                ▼
                    MODEL 3 FEDERATION LAYER
              adapters • authentication • normalization
                                │
                                ▼
                    MODEL 1 CAMERA REGISTRY
                    (PostgreSQL + PostGIS + GIS)
                                │
                    ┌───────────┴───────────────────────┐
                    ▼                                   ▼
            MODEL2 STREAM INGESTION                  METADATA/EVENTS
             (FFmpeg/GStreamer)                       (Kafka/API)
                    │                                   │
                    ▼                                   │
               AI PIPELINE                              │
 Detection → Tracking → Plate → ANPR → Consensus        │
                    │                                   │
                    └───────────┬───────────────────────┘
                                ▼
                     OBSERVED VEHICLE LAYER
                                │
                 ┌──────────────┼──────────────┐
                 ▼              ▼              ▼
             Watchlist        Journey        Analytics
              Matcher         Engine          / Search
                 │              │              │
                 └──────────────┼──────────────┘
                                ▼
                         FastAPI Services
                                │
                                ▼
                      REACT COMMAND CENTRE
               GIS • Video Wall • Vehicles • Alerts

## 4.1 Workflow-to-Architecture Model Mapping

The proposed solution follows a hybrid architecture combining Model 1 (Centralized CCTV Registry and GIS), Model 3 (Federation/Middleware Layer), and selective Model 2 (Direct Stream Integration). The following mapping shows how each implementation step contributes to the overall architecture.

Model 1 provides the centralized registry, metadata, spatial intelligence and governance foundation. Model 3 acts as the primary federation and event-correlation backbone for heterogeneous departmental CCTV systems. Model 2 capabilities are used selectively where direct RTSP, ONVIF, WebRTC or compatible stream access is available. The AI analytics stages operate as platform services over the integrated streams rather than representing separate architectural models.

| Step | Workflow | Architectural Model / Layer | Role |
| --- | --- | --- | --- |
| 1 | Sentinel access | Model 3 + selective Model 2 | Establish access to Sentinel RTSP/WebRTC/HLS interfaces through the federation/integration layer; direct stream access is used where applicable. |
| 2 | Catalogue ingestion | Model 1 | Register cameras, departments, locations, stream endpoints and metadata in the centralized CCTV registry. |
| 3 | RTSP + OpenCV + PTS | Model 3 + selective Model 2 | Federation connectors provide stream access; direct RTSP ingestion is used where supported, with OpenCV decoding and PTS-based timing. |
| 4 | Vehicle detection + tracking | Platform AI Layer | Detect and track vehicles on integrated CCTV streams. |
| 5 | Best frames + plate detection | Platform AI Layer | Select high-quality vehicle frames and detect license-plate regions. |
| 6 | ANPR/OCR + multi-frame consensus | Platform AI Layer | Extract registration numbers and improve recognition reliability using multi-frame evidence. |
| 7 | Observed vehicle database | Model 1 + Model 3 | Centrally store normalized vehicle observations and associated events/evidence. |
| 8 | Representative synthetic watchlist | Model 1 | Maintain centralized watchlist records and associated metadata. |
| 9 | Real-time watchlist matching | Model 3 | Correlate incoming vehicle events against active watchlist entries and generate matches. |
| 10 | Kafka event pipeline | Model 3 | Provide asynchronous event transport between analytics, correlation, watchlist and alert services. |
| 11 | Cross-camera vehicle correlation | Model 3 | Correlate vehicle observations across cameras and departments. |
| 12 | Vehicle journey reconstruction | Model 3 + Model 1 | Correlate observations while using centralized camera/location metadata for journey reconstruction. |
| 13 | PostgreSQL/PostGIS + GIS | Model 1 | Provide centralized persistence, camera registry, spatial data and GIS-based intelligence. |
| 14 | React command centre | Model 1 + Model 3 + selective Model 2 | Provide unified access to camera registry, GIS, live video, analytics, events, watchlists and alerts. |
| 15 | Security/RBAC/audit | Cross-cutting | Apply authentication, authorization, audit logging, encryption and governance across all architectural components. |

Architectural interpretation: Model 3 serves as the primary integration and federation backbone of the proposed system. Selective Model 2 capabilities are incorporated as direct stream connectors where existing cameras or VMS platforms provide suitable RTSP, ONVIF, WebRTC or compatible interfaces. Therefore, Model 2 is not treated as a separate architectural layer; rather, its direct-integration capability is utilized within the broader Model 3 federation architecture.

## 4.2 End-to-End Workflow

Onboard or import the camera metadata into the Model 1 registry.

Select an authorized integration method through Model 3 adapters or Model 2 direct streaming.

Validate stream connectivity and camera health.

Decode live media using FFmpeg/GStreamer/OpenCV.

Detect and track vehicles using AI.

Select high-quality frames and detect number plates.

Run ANPR/OCR and multi-frame consensus.

Persist evidence-backed observations.

Publish normalized events through Kafka.

Compare eligible recognized vehicles with active watchlists.

Correlate observations across cameras using a validated common time basis where available.

Present results through FastAPI and the React command centre.

## 4.3 Component Interaction and Data Flow

The platform separates source integration, media processing, intelligence, persistence and operator interaction into independently scalable components. Camera/VMS-specific connectivity is isolated in Model 3 adapters, while Model 1 maintains the authoritative camera identity and spatial metadata.

Component interaction flow:

Department Cameras / NVRs / VMS
                |
                v
      Model 3 Adapters / Connectors
                |
                +---- Camera metadata / health ----> Model 1 Registry + PostGIS
                |
                v
        Stream Ingestion Workers
                |
                v
       AI Scheduler / Decoder
                |
                +----> Detection / Tracking / ANPR
                |
                v
       Normalized Events + Evidence
                |
             Apache Kafka
                |
      +---------+---------+
      |                   |
 Watchlist Matcher   Cross-Camera Correlator
      |                   |
      +---------+---------+
                v
          Alert / Journey Services
                |
                v
        FastAPI + PostgreSQL/PostGIS
                |
                v
          React Command Centre

## 4.4 Departmental Infrastructure Retention and Central Intelligence Layer

The platform follows an overlay approach: departmental cameras, NVRs, VMS platforms and existing recording arrangements remain operationally owned by the respective departments. The statewide platform adds a secure integration and intelligence layer rather than requiring immediate rip-and-replace of existing infrastructure.

Department / Source Domain

Cameras -> NVRs -> Existing VMS / Storage
                       |
                       | authorized access
                       v
                 Model 3 Connector
                       |
                       v
         STATE CCTV INTELLIGENCE LAYER
 Camera Registry | Stream Integration | AI | Kafka
 Watchlists | Correlation | Alerts | GIS | APIs
                       |
                       v
    Police Command Centre

# 5. Camera Registry and GIS Foundation

The registry is the integration control plane. Camera identifiers must come from the authoritative source catalogue or authorized departmental inventory; the application must not invent IDs or assume a fixed camera set.

| Metadata | Examples | Operational Use |
| --- | --- | --- |
| Identity | camera_id, name, number | Stable integration key |
| Ownership | department, owner | Access policy and accountability |
| Location | address, latitude, longitude | GIS and spatial operations |
| Technical | codec, width, height, bitrate, FPS | Decoder/worker sizing; FPS informational only |
| Connectivity | RTSP, HLS, WebRTC, VMS/API | Connector selection |
| Health | status, last_seen, errors, reconnects | Operational monitoring |
| Timing | timezone, source-time metadata, loop/session | Temporal correlation |
| Governance | retention, permitted analytics | Policy enforcement |

PostGIS stores verified camera locations as geometry(Point,4326). Cameras without verified coordinates remain non-spatial and are clearly marked as such. The platform does not fabricate coordinates.

# 6. CCTV Camera, NVR and VMS Interoperability

| Source | Mechanism | Design |
| --- | --- | --- |
| Department VMS | Vendor API/SDK | Model 3 adapter isolates vendor specifics |
| IP camera | RTSP/ONVIF | Direct authorized access; suitable for AI |
| Restricted-network feed | HLS | Dashboard/remote use where authorized |
| Low-latency browser preview | WebRTC/WHEP | Operator viewing; browser never consumes RTSP directly |
| Future vendor | Plugin/adapter interface | Connector contract avoids central rewrites |

The interoperability layer explicitly covers cameras, NVRs and VMS platforms. Integration is capability-driven: the platform discovers or receives supported endpoints, credentials, media properties and control-plane capabilities and selects the appropriate connector.

| Source type | Typical integration mechanism | Platform treatment |
| --- | --- | --- |
| IP camera | RTSP / ONVIF | Direct authorized access through a Model 3 stream connector; AI-ready. |
| NVR | RTSP / ONVIF / NVR API | NVR adapter exposes camera channels, media capabilities and health without replacing the NVR. |
| Department VMS | Vendor API / SDK / RTSP gateway | Model 3 adapter isolates vendor specifics and normalizes metadata/events. |
| Legacy / analog camera | Encoder / gateway | Gateway converts legacy camera output into an authorized digital stream. |
| Restricted or remote source | HLS | Remote/browser-safe visualization where authorized. |
| Low-latency operator preview | WebRTC / WHEP | Used for browser viewing; RTSP is not exposed directly to the browser. |

## 6.1 Integration Contract

Each adapter should expose: camera identity, capabilities, authorized stream URLs, health state, metadata/events, connection errors and a normalized internal representation.

## 6.2 Rules

Use the camera catalogue as the authoritative list.

Force RTSP over TCP for AI clients.

Drive timing from media PTS rather than CAP_PROP_FPS or frame-arrival time.

Tolerate H.264/H.265, mixed resolutions and irregular frame intervals.

Use exponential backoff for reconnection.

Treat join-time decoder warnings as non-fatal when the stream recovers.

Expect looping/scene discontinuities and reset long-lived track state appropriately.

Consume only; do not publish to the gateway or invoke control APIs.

# 7. Stream Ingestion and Processing

Camera Source → Connector → RTSP/HLS/WebRTC → Decoder → Frame Buffer
                                             │
                                             ▼
                                    AI Worker Scheduler
                                             │
                       ┌─────────────────────┼─────────────────────┐
                       ▼                     ▼                     ▼
                  Detection              Tracking             Health/PTS
                       │                     │                     │
                       └─────────────────────┼─────────────────────┘
                                             ▼
                                      Vehicle Track

## 7.1 Continuous Coverage vs AI Sampling

A critical distinction is made between stream coverage and expensive inference frequency. For designated-vehicle tracing, participating cameras should have continuous stream coverage over the evaluation window. The detector may run at a controlled sampling interval while the tracker handles intermediate frames. ANPR/OCR is triggered only when useful plate opportunities occur.

## 7.2 Timing

media_pts_ms             = media timeline
source_time              = common calendar time, only if validated
ingested_at_utc          = application processing time

Never: event_time = datetime.now()
Never: event_time = capture_start + frame_number / FPS

# 8. AI Analytics Pipeline

Decoded Frame
     ↓
Vehicle Detection
     ↓
Multi-Object Tracking
     ↓
Best-Frame Selection
     ↓
Plate Detection
     ↓
Plate Quality
     ↓
ANPR / OCR
     ↓
Multi-Frame Consensus
     ↓
Vehicle Observation

| Stage | Output | Design Rationale |
| --- | --- | --- |
| Vehicle detection | class, bbox, confidence | Detect all sufficiently visible vehicles, not only watchlist vehicles |
| Tracking | track_id, first/last PTS | Associate frames belonging to same camera-local track |
| Best-frame selection | top-N frames | Reduce OCR noise and compute |
| Plate detection | plate bbox/confidence | Focus OCR on plate ROI |
| Plate quality | sharpness, area, brightness, contrast | Rank usable candidates |
| ANPR/OCR | raw/normalized text | Recognize registration |
| Consensus | final state + evidence | Reduce single-frame OCR errors |

## 8.1 AI Analytics Capability

The current implementation focuses on vehicle intelligence and ANPR. The architecture is intentionally extensible so that person, face and general object analytics can be added as dedicated services without changing the CCTV integration backbone. Only analytics validated in the PoC are presented as implemented.

| Analytics capability | PoC status | Production direction | Typical output |
| --- | --- | --- | --- |
| Vehicle detection | Implemented | Distributed AI workers | class, bbox, confidence |
| Vehicle tracking | Implemented | Distributed tracking workers | track_id, trajectory, first/last PTS |
| Plate detection | Implemented | Distributed plate detector | plate bbox, confidence |
| ANPR / OCR | Implemented | Model-managed ANPR workers | raw/normalized text, confidence |
| Multi-frame consensus | Implemented | Evidence-backed recognition service | CONFIRMED / PROBABLE / UNCERTAIN / UNREADABLE |
| Person detection | Architecture-ready | Distributed person detection | person bbox, confidence |
| Person tracking | Architecture-ready | Cross-frame person tracking | track_id, trajectory |
| Facial Recognition System (FRS) | Future / policy-controlled | Dedicated face embedding and matching service | candidate match + confidence |
| General object detection | Architecture-ready | Pluggable analytics models | object class, bbox, confidence |
| Intrusion / loitering / occupancy | Future extension | Rule + AI event services | policy-specific event |

# 9. ANPR/OCR and Multi-Frame Consensus

The ANPR subsystem does not treat one OCR result as truth. It aggregates independent video-frame observations for a track and weighs detection confidence, plate quality, OCR confidence and registration-format validity.

Frame 1 → GJ01AB1234
Frame 2 → GJ01A81234
Frame 3 → GJ01AB1234
Frame 4 → GJ01AB1234
                 ↓
          Multi-frame consensus
                 ↓
          GJ01AB1234 / CONFIRMED

Preserve raw OCR and normalized OCR separately.

Do not blindly substitute ambiguous characters such as O/0 or I/1.

Distinguish plate detector confidence from OCR confidence and plate quality.

Treat preprocessing variants of the same physical frame as weaker evidence than genuinely different frames.

Return CONFIRMED, PROBABLE, UNCERTAIN or UNREADABLE rather than forcing a value.

Retain the complete evidence chain from final text to camera, track, frame, PTS and crop.

## 9.1 AI Evidence Chain

AI evidence should remain traceable from source media to operator action:

Camera
  -> Frame / PTS
  -> Vehicle track
  -> Plate crop
  -> OCR candidates
  -> Multi-frame consensus
  -> Vehicle observation
  -> Watchlist match
  -> Alert
  -> Operator acknowledgement / escalation / resolution

Every decision should retain camera_id, track_id or frame reference, PTS, source-time status, confidence values and evidence reference. This allows an alert to be reviewed back to the original observation without treating OCR text or an alert as an unqualified source of truth.

# 10. Observed Vehicle Intelligence Layer

The Observed Vehicle DB answers “What did the CCTV system observe?” It is deliberately separate from watchlists.

| Entity | Purpose |
| --- | --- |
| observed_vehicles | One aggregate record per normalized observed registration |
| vehicle_observations | Individual camera/track observations with PTS/time/evidence |
| vehicle_tracks | Camera-local track segments |
| anpr_observations | Raw OCR/evidence records |

# 11. Watchlist Architecture and Matching

The current PoC uses a synthetic representative watchlist. Some synthetic entries intentionally correspond to observed registrations from the CCTV data so that the demonstration produces deterministic positive matches; other records are non-matches. Every synthetic entry is explicitly marked SYNTHETIC_DEMO.

Eg:
   Observed Vehicle DB
   GJ01AB1234
   GJ05CD5678
   GJ18XY9087
        │
        ▼
Active Watchlist
   GJ01AB1234 → HIGH → SYNTHETIC_DEMO
   GJ18XY9087 → LOW → SYNTHETIC_DEMO
   GJ99ZZ9999 → MEDIUM → SYNTHETIC_DEMO
        │
        ▼
Exact normalized matching

Only ACTIVE watchlist entries are eligible.

Exact normalized equality is the primary automated decision.

UNCERTAIN/PROBABLE recognition is not silently converted into a confirmed high-priority alert.

Fuzzy similarity may produce POSSIBLE_MATCH_REVIEW, not automatic confirmation, unless policy explicitly permits it.

Repeated detections from the same track are deduplicated to avoid alert storms.

# 12. Kafka Event Pipeline

vehicle.detections → vehicle.anpr → watchlist.matches → alerts
                              │
                              └────→ journey / analytics consumers

| Topic | Purpose |
| --- | --- |
| vehicle.detections | Vehicle detection/track events |
| vehicle.anpr | Recognized registration events |
| watchlist.matches | Watchlist decision events |
| alerts | Operator-facing alert-ready events |
| vehicle.events.dlq | Non-recoverable malformed/failing events |

Use event_id and schema_version in a common envelope.

Keep matching logic in the matcher; Kafka is transport, not business logic.

Use retries and bounded backoff for transient failures.

Use a DLQ for malformed/non-recoverable events.

Use idempotency to tolerate duplicate Kafka delivery.

Do not send raw video or large image binaries through Kafka; use evidence references.

Use meaningful message keys to preserve useful ordering for related events.

# 13. Alert Generation and Notification Workflow

Alert generation is separated from raw detection so that recognition confidence, watchlist policy, alert severity, deduplication and operator workflow can be controlled independently. The alert service consumes normalized match events and produces operator-facing alerts only when configured policy criteria are satisfied.

Alert generation workflow

AI / ANPR Event
      |
      v
Watchlist / Rule Evaluation
      |
      +---- No match ----> Store observation
      |
      +---- Match -------> Confidence + policy check
                               |
                               v
                         Deduplication
                               |
                               v
                       Alert Classification
                  +------------+------------+
                  |            |            |
                CRITICAL      HIGH       MEDIUM / LOW
                  |            |            |
                  +------------+------------+
                               |
                               v
                          Kafka alerts
                               |
                               v
                        Alert Service / API
                               |
             +-----------------+-----------------+
             |                 |                 |
        Command Centre     Audit Log       Notifications
             |
             v
     Acknowledge / Escalate / Resolve / Dismiss

## 13.1 Alert Rules and Eligibility

Only eligible, policy-authorized watchlist matches can create automated alerts. CONFIRMED recognition is the default basis for automatic high-priority alerting; PROBABLE or UNCERTAIN results remain reviewable unless an explicit policy permits otherwise. Repeated detections from the same track are deduplicated within a configurable window.

## 13.2 Alert Prioritization

| Priority | Typical trigger | Operator treatment |
| --- | --- | --- |
| CRITICAL | Confirmed high-priority watchlist match or policy-defined urgent event | Prominent alert, immediate review, optional external notification subject to policy. |
| HIGH | Confirmed watchlist match requiring prompt action | Dashboard priority, configured sound/visual cue, acknowledge and investigate. |
| MEDIUM | Lower-risk or review-required match | Queue for operator review; evidence available on demand. |
| LOW | Informational / non-urgent event | Retained for audit and search; no intrusive notification by default. |

## 13.3 Alert Lifecycle and Operator Interaction

| State | Meaning |
| --- | --- |
| NEW | Alert created and awaiting operator attention. |
| ACKNOWLEDGED | Operator has taken ownership of the alert. |
| UNDER_REVIEW | Evidence, camera history and related observations are being examined. |
| ESCALATED | Alert transferred to a higher authority or response workflow. |
| RESOLVED | Investigation/action completed with an auditable outcome. |
| DISMISSED | Reviewed and closed as non-actionable, with reason recorded. |

## 13.4 Notification

The command centre presents alert priority, matched watchlist entry, registration or subject, camera, observation time basis, confidence, evidence crop and related observations. Notification channels may include in-platform alerts and, where authorized, email/SMS/push integrations. All alert actions and configuration changes are auditable.

# 14. Cross-Camera Correlation and Journey Reconstruction

The primary cross-camera identity key is normalized registration number, but every camera/track observation remains a distinct evidence record. Camera-local PTS is never assumed to be a global clock.

## 14.1 Time Model

camera-local PTS  ≠ global time
source_time       = global calendar reference only when validated
ingested_at_utc   = application processing time

## 14.2 Journey States

| State | Meaning |
| --- | --- |
| OBSERVATION_SEQUENCE_ONLY | Camera-local PTS available; common source time unresolved |
| PARTIAL_JOURNEY | Some temporal relationships can be established |
| JOURNEY_RECONSTRUCTED | Common source-time basis plus required metadata supports ordering |
| TIME_CONFLICT | Reliable temporal information produces a conflict requiring review |

## 14.3 Spatial Correlation

When verified camera coordinates are available, the system can calculate straight-line distance between observation cameras. This is a spatial plausibility indicator and is not the vehicle’s actual road distance or exact route.

## 14.4 Observation Graph

GJ01AB1234
   │
   ├── CAM01 / TRK-0001
   │
   ├── CAM07 / TRK-0019
   │
   └── CAM10 / TRK-0042

# 15. PostgreSQL/PostGIS Data Architecture

| Table | Primary Purpose | Key Fields |
| --- | --- | --- |
| cameras | Registry/GIS | camera_id, metadata, geom |
| observed_vehicles | Aggregate observed identity | vehicle_id, normalized_registration_number |
| vehicle_observations | Observation evidence | camera_id, track_id, PTS, source_time, confidence, evidence |
| vehicle_tracks | Track segments | track_id, camera_id, first/last PTS |
| anpr_observations | Raw OCR evidence | frame_id, raw/normalized text, confidence, crop |
| watchlist_entries | Monitoring entries | watchlist_id, normalized registration, category, priority, status |
| watchlist_matches | Match decisions | match_id, observation_id, watchlist_id, decision |
| alerts | Operator workflow | alert_id, match_id, priority, status |
| vehicle_journeys | Journey state | journey_id, vehicle_id, time_basis, status |
| journey_observations | Ordered observations | journey_id, observation_id, sequence |
| journey_legs | Relationships | from/to observation, time delta, distance, plausibility |

Use geometry(Point,4326) and a GiST index for verified camera coordinates. Use JSONB only for flexible metadata/evidence extension fields. Do not store raw video in PostgreSQL.

# 16. React Command Centre

Dashboard | Cameras | Video Wall | Vehicles | Watchlist | Alerts
                                      │
                           Vehicle Investigation
                                      │
                    Observations → Evidence → Journey

| View | Core Capability |
| --- | --- |
| Dashboard | Cameras/vehicles/ANPR/alerts KPIs and recent events |
| Camera Map | Verified camera points, status and filters |
| Video Wall | 1/4/9 selected cameras; WebRTC/HLS preview |
| Vehicle Search | Registration search and evidence history |
| Journey | Cross-camera observations and GIS when valid |
| Watchlist | Active synthetic/authorized entries |
| Alerts | Priority/status filtering, acknowledgement and evidence |
| Camera Details | Health and technical metadata |

The browser never connects directly to PostgreSQL or Kafka and should not consume RTSP directly. React consumes FastAPI APIs and, where available, browser-safe WebRTC/HLS preview endpoints.

The command centre alert view should provide priority and state filters, acknowledgement, escalation, resolution/dismissal actions, evidence review, related observations and an audit trail. This turns alert generation into an operational workflow rather than a passive event list.

# 17. APIs and Integration Contracts

| Endpoint | Method | Purpose |
| --- | --- | --- |
| /api/cameras | GET | List/filter cameras |
| /api/cameras/{id} | GET | Camera details |
| /api/cameras/{id}/health | GET | Health |
| /api/vehicles/{registration} | GET | Vehicle summary |
| /api/vehicles/{registration}/observations | GET | Observation history |
| /api/vehicles/{registration}/journey | GET | Journey/sequence |
| /api/vehicles/{registration}/geojson | GET | GIS-ready data |
| /api/watchlist | GET | Watchlist listing subject to authorization |
| /api/alerts | GET | Alert listing/filtering |
| /api/alerts/{id}/acknowledge | POST | Alert action |

API responses must expose source_time, source_time_status, recognition_pts_ms and ingested_at_utc separately. The frontend must not infer CCTV event time from ingestion time.

# 18. Security, Privacy, RBAC and Auditability

| Control | Implementation |
| --- | --- |
| Authentication | Central identity/authentication layer; future integration with departmental SSO |
| Authorization | Role/department-based access to cameras, evidence, watchlists and alerts |
| Network segmentation | Separate camera/VMS, AI, event, database and management planes |
| Secrets | Environment/secret manager; never log passwords/cookies/tokens |
| Transport security | TLS for management/API traffic; controlled media access |
| Database security | Least privilege, parameterized queries, encrypted backups |
| Audit | Access logs, configuration changes, watchlist changes, match/alert actions |
| Privacy | Purpose limitation, policy-defined retention, access control and audit logging |

# 19. Network and Bandwidth Planning

Bandwidth must be planned from measured per-camera bitrate and selected camera count, not a single assumed stream rate. The architecture distinguishes AI ingestion, dashboard viewing and control-plane traffic.

AI ingress ≈ Σ active-camera bitrates
Dashboard bandwidth ≈ Σ selected video-wall streams
Metadata/events ≪ raw-video bandwidth
Control plane = catalogue + health + API + Kafka

Use RTSP over TCP for AI where reachable.

Use HLS for remote/restricted networks where authorized.

Use WebRTC for low-latency browser preview.

Use regional/edge processing concepts as camera count grows to reduce long-haul raw-video transport.

Use bounded buffers and backpressure to avoid memory growth under downstream slowdown.

# 20. Storage and Retention Strategy

| Data | Strategy |
| --- | --- |
| Camera metadata | Long-lived, versioned and audited |
| Full video | Remain with departmental/VMS systems unless centralized retention is explicitly required |
| Vehicle observations | Indexed metadata; policy-defined retention |
| ANPR evidence | Short/medium retention based on policy and investigation requirements |
| Watchlist/matches | Access-controlled; retain audit history |
| Journey reports | Policy-defined; preserve final audit output where required |

The architecture supports hot/warm/cold retention tiers in a future production environment, but the PoC avoid unnecessary raw-video duplication.

# 21. Scalability to Approximately 80,000 Cameras

The statewide target is addressed by horizontal scaling and regionalization rather than by placing all 80,000 streams on one server.

~80,000 CAMERAS
                           │
          ┌────────────────┼────────────────┐
          ▼                ▼                ▼
      Region A         Region B         Region C
     connectors        connectors       connectors
          │                │                │
          ▼                ▼                ▼
       AI workers       AI workers       AI workers
          └────────────────┼────────────────┘
                           ▼
                         Kafka
                           ▼
              central event / API services
                           ▼
                    PostgreSQL/PostGIS

Scale stream/AI workers horizontally.

Partition Kafka topics according to event volume and ordering requirements.

Use per-camera metadata to size decoder/buffer/AI resources.

Centralize metadata and events while keeping raw video closer to source where practical.

Use health-aware scheduling and controlled camera activation.

Benchmark small batches (1, 5, 10, 30 cameras) before extrapolating statewide capacity.

## 21.1 Capacity and Scaling Model

The statewide target is a registry scale of approximately 80,000 cameras, not an assumption that all 80,000 cameras must be simultaneously decoded and AI-inferred on a single platform tier. The architecture separates registry-only, health monitoring, operator-viewing and AI-active workloads so compute is allocated according to operational need.

| Deployment profile | Typical activity | Scaling implication |
| --- | --- | --- |
| Registry-only | Metadata, ownership, GIS, health state | Minimal media compute; registry/API capacity is the primary concern. |
| Health monitoring | Lightweight connectivity/health checks | Scales with connector count and check frequency. |
| Operator viewing | Selected cameras on demand | Browser bandwidth scales with selected streams, not registered inventory. |
| AI-active | Continuous or scheduled inference on selected cameras | Primary CPU/GPU sizing driver; scale AI workers horizontally. |
| Investigation burst | Temporary intensive analytics | Use elastic/on-demand workers and bounded queues. |

Capacity planning formulas

Total AI ingress ~= sum of bitrates of active AI streams.
Required AI workers ~= active AI streams / measured streams-per-worker.
Event throughput ~= active cameras x measured event rate x average event size.
Observation storage ~= observations per day x retention period x average record size.

Final statewide sizing must be derived from measured PoC benchmarks, actual per-camera bitrate/resolution, chosen AI sampling/inference rate, retention policy and regional network constraints. The HLD therefore avoids unsupported fixed hardware claims.

## 21.2 Geo-Distributed Regional Processing

To support geographically dispersed cameras, connector and AI workloads should be placed in regional processing zones close to camera/VMS networks. Central services should primarily consume normalized metadata, events and selected evidence references, reducing dependence on one statewide raw-video path.

Regional processing topology

                    STATE CCTV PLATFORM
                           |
          +----------------+----------------+
          |                |                |
     REGION NORTH     REGION CENTRAL    REGION SOUTH
       Connectors       Connectors        Connectors
          |                |                |
       AI Workers       AI Workers       AI Workers
          |                |                |
          +----------------+----------------+
                           |
                         Kafka
                           |
                 Central Event / API Layer
                           |
                    PostgreSQL/PostGIS
                           |
                    Command Centre

# 22. Reliability, Disaster Recovery and Business Continuity

Exponential stream reconnection: 2, 4, 8, 16 and 30 seconds maximum.

Kafka retries with DLQ for non-recoverable processing failures.

Idempotent consumers for duplicate event delivery.

PostgreSQL scheduled backups and point-in-time recovery in production.

Health monitoring for cameras, adapters, workers, Kafka consumers and database.

Department/region isolation so one failing source domain does not take down the central platform.

Optional active-passive or clustered services for critical control-centre components.

# 23. Deployment Architecture

| Layer | PoC | Production Direction |
| --- | --- | --- |
| Integration | Python connector services | Regional connector/adapter pools |
| AI | GPU-enabled Python/PyTorch workers where available | Distributed AI workers |
| API | FastAPI | Horizontally scaled FastAPI services |
| Events | Kafka | Multi-broker Kafka cluster |
| DB | PostgreSQL/PostGIS | HA PostgreSQL/PostGIS |
| Frontend | React | HA web tier |
| Evidence | Filesystem/reference paths | Policy-controlled object/evidence storage |

# 24. Performance and Capacity Planning

| Metric | Measurement |
| --- | --- |
| Stream connection latency | Connect request to first valid decoded frame |
| AI inference latency | Frame/crop inference time |
| ANPR latency | Plate crop to OCR result |
| Match latency | ANPR event to watchlist decision |
| Alert latency | Match to alert event |
| Kafka lag | Consumer lag under load |
| DB latency | Vehicle lookup/journey/spatial query |
| Reconnect recovery | Disconnect to recovered stream |

All performance claims must be backed by measured PoC runs. Camera count, GPU capacity, bitrate and inference sampling must be recorded for reproducibility.

# 25. Testing and Evaluation Strategy

| Test Layer | Focus |
| --- | --- |
| Unit | Normalization, detection/tracking schemas, consensus, matching, repositories |
| Stream | RTSP/TCP, codecs, reconnects, PTS, discontinuities |
| ANPR | Clear/night/blur/glare/small/angled conditions |
| Data | Idempotency, provenance, duplicate prevention |
| Kafka | Producer/consumer, retry, DLQ, duplicate event |
| Database | Migration, indexes, spatial queries, GeoJSON |
| UI | Camera/video/vehicle/journey/watchlist/alert workflows |
| Load | 1→5→10→30 camera tests |
| Security | RBAC, authorization, secret leakage, audit |

# 26. PoC and Government Feed Demonstration

## 26.1 Participant-Owned Demonstration

Use a participant-controlled feed or controlled dataset to demonstrate the complete end-to-end product: onboarding, AI detection, ANPR, representative synthetic watchlist correlation, automatic match/alert and operator visualization. This demonstrates product functionality under controlled conditions.

## 26.2 Government-Provided Feed Demonstration

- Open the platform camera registry and select the government-provided feed/camera.

- Show the actual authorized government feed inside the platform.

- Show camera metadata and health state.

- Show AI analytics output on the feed where feasible: vehicle/plate detection and ANPR.

- Show the correct media/source timestamp basis; do not display application ingestion time as CCTV event time.

- Show the generated observation/report with camera, track/frame, plate, confidence, PTS/source-time status and evidence.

- For the designated-vehicle test, ensure relevant participating cameras remain continuously covered during the evaluation window so a vehicle that appears briefly is not missed simply because a camera was not being processed.

## 26.3 Supporting Technical Evidence

Output report in JSON/CSV/PDF as appropriate.

Example evidence crops and observation IDs.

Stream/AI performance logs.

Architecture diagrams and test summary.

Hosted platform URL and credentials if permitted.

# 27. Technical Prerequisites and Department Inputs

| Input | Examples | Purpose |
| --- | --- | --- |
| Camera inventory | ID, name, department, location | Registry |
| Stream access | RTSP/ONVIF/API/HLS credentials/permissions | Integration |
| VMS details | Vendor, model/version, API/SDK | Adapter selection |
| Network | IP, ports, NAT/firewall rules | Connectivity |
| Media properties | Codec, resolution, bitrate | Worker sizing |
| Time metadata | Timezone, source timestamp, synchronization source | Cross-camera ordering |
| Coordinates | Verified latitude/longitude/address | GIS |
| Retention | Policy and duration | Storage planning |
| Analytics policy | Allowed analytics/watchlist classes | Governance |
| Database integration | Authorized VAHAN/SARTHI/eGujCop/AFIS/NAFIS APIs | Future correlation |

## 27.1 Department Integration Readiness Checklist

Before onboarding a department, the following information should be collected and validated so connector selection, security approval, timing semantics, GIS mapping and AI feasibility can be assessed consistently.

| Readiness domain | Required information |
| --- | --- |
| CCTV / NVR / VMS | Manufacturer, model, firmware/version, channel count, ONVIF profile, API/SDK availability, concurrent-client limits. |
| Stream access | RTSP/ONVIF/API/HLS/WebRTC endpoints, credentials or service accounts, authorization scope and test access window. |
| Network | IP/FQDN, ports, routing/VPN, NAT/firewall policy, bandwidth, latency/packet-loss expectations and certificate requirements. |
| Media properties | Codec, resolution, bitrate, frame characteristics, keyframe behavior and night/low-light conditions. |
| Time synchronization | Timezone, NTP source, source timestamp availability, clock drift information and media PTS behavior. |
| GIS / location | Verified latitude/longitude or authoritative address, camera mounting context and coverage direction where available. |
| Governance | Retention period, permitted analytics, access classification, department ownership and approval workflow. |
| Database / API integration | Authorized interfaces to relevant departmental databases such as VAHAN/SARTHI/eGujCop/AFIS/NAFIS where legally and technically permitted. |
| Operational contacts | Department technical owner, security/network contact and incident/escalation contact. |

# 28. Risks and Mitigations

| Risk | Mitigation |
| --- | --- |
| Poor plate quality | Best-frame selection, plate quality scoring, multi-frame consensus |
| Missed transient vehicles | Continuous camera coverage; controlled detection sampling rather than sparse snapshots |
| Camera-local clocks | Source-time resolver; never compare raw PTS across cameras |
| Missing coordinates | Verified metadata only; explicit non-spatial state |
| VMS heterogeneity | Model 3 adapter/plugin architecture |
| Duplicate alerts | Track-level deduplication + event idempotency |
| Small evaluation dataset | Development and held-out datasets; report sample counts |
| Network/stream failure | TCP, health monitoring, reconnect backoff, HLS fallback where authorized |
| False recognition | Conservative confidence/consensus states and evidence-backed decisions |

# 29. Expected Operational Benefits

Unified statewide camera inventory and GIS visibility.

Integration of heterogeneous departmental VMS without immediate replacement.

Faster vehicle investigation through ANPR and evidence-backed search.

Real-time correlation of recognized entities with authorized watchlists.

Cross-camera observation history for vehicle investigations.

Operational health visibility across cameras and integrations.

Event-driven architecture suitable for incremental onboarding.

Auditable AI evidence chain from camera frame to match/alert.

Clear migration path from PoC JSON/JSONL stores to PostgreSQL/PostGIS.

# 30. Conclusion

The proposed Hybrid Model 5 architecture combines the mandatory Model 1 registry with Model 3 federation and selective Model 2 direct stream integration. This approach prioritizes interoperability, operational continuity and incremental adoption while supporting AI-powered analytics and intelligent alerts.

The platform is designed around evidence integrity: vehicle identity, plate recognition, watchlist matching and cross-camera journey claims remain separate decisions with explicit confidence and provenance. The architecture scales by adding integration and AI workers, partitions event workloads through Kafka, persists operational intelligence in PostgreSQL/PostGIS, and presents a unified command-centre workflow through React.